EGR 280 – Mechanics Problem Set 1

Graphics and problem statements © 2004 R.C. Hibbeler.

Published by Pearson Education, Inc., Upper Saddle River, NJ.

|  | 1.1 Determine the magnitude of the resultant force and its direction, measured counter-clockwise from the positive x-axis. Ans: R = 393 lb, θ = 353° |
| --- | --- |
|  | 1.2 Determine the magnitude of the resultant force and its direction, measured counter-clockwise from the positive x-axis. Ans: R = 998 N, θ = 135° |
|  | 1.3 Two forces are applied at the end of a screw eye in order to remove the post. Determine the angle θ (0° ≤ θ ≤ 90°) and the magnitude of the force F so that the resultant force acting on the post is directed vertically upward and has a magnitude of 750 N. Ans: θ = 18.6°, F = 319 N |
|  | 1.4 Determine the magnitudes of F1 and F2 so that the particle is in equilibrium. Ans: F1 = 435 lb, F2 = 171 lb |
|  | 1.5 Determine the magnitude and direction of F1 so that particle P is in equilibrium. Ans: θ = 12.9°, F1 = 552 N |
|  | 1.6 Determine the maximum weight of the flowerpot that can be supported without exceeding a cable tension of 50 lb in either cable AB or AC. Ans: W = 76.6 lb |