EGR 280 – Mechanics Problem Set 16

Graphics and problem statements © 2004 R.C. Hibbeler.

Published by Pearson Education, Inc., Upper Saddle River, NJ.

|  | 16.1 The 2-lb bottle rests on the check-out conveyor at a grocery store. If the coefficient of static friction is μs = 0.2, determine the largest acceleration the conveyor can have without causing the bottle to slip or tip. The center of gravity is at G. Ans: aG = 6.04 ft/s2 |
| --- | --- |
|  | 16.2 The sports car has a mass of 1.5 Mg and a center of gravity at G. Determine the shortest time it takes for it to reach a speed of 80 km/hr, starting from rest, if the engine drives only the rear wheels. μs = 0.2. Neglect the mass of the wheels. If power can be supplied to all wheels, what would be the shortest time for the car to reach 80 km/hr? Ans: rear wheels: t = 17.5 s 4 wheels: t = 11.3 s. |
|  | 16.3 The drum truck supports the 600-lb drum that has a center of gravity at G. If the operator pushes it forward with a horizontal force of 20 lb, determine the acceleration of the truck and the normal reactions at each of the four wheels. Neglect the mass of the wheels. Ans: aG = 1.07 ft/s2, NB = 86.7 lb, NA = 213 lb |
|  | 16.4 The 80-kg disk is supported by a pin at A. If it is released from rest from the position shown, determine the initial horizontal and vertical reactions at the pin. Ans: Ax = 0, Ay = 262 N |
|  | 16.5 The disk has a mass M and a radius R. If a block of mass m is attached to the cord, determine the angular acceleration of the disk when the block is released from rest. Also, what is the velocity of the block after it falls a distance of 2R starting from rest? Ans: α = 2mg/[R(M + 2m)] v2 = 8mgR/(M + 2m) |
|  | 16.6 Disk D turns with a constant clockwise angular velocity of 30 rad/s. Disk E has a weight of 60 lb and is initially at rest when it is brought into contact with D. If μk = 0.3, determine the time required for disk E to attain the same angular velocity as disk D. Neglect the weight of bar BC. Ans: t = 1.09 s |
|  | 16.7 The 2-kg slender bar is supported by cord BC and then released from rest at A. Determine the initial angular acceleration of the bar and the tension in the cord. Ans: α = 28.0 rad/s2, T = 5.61 N |
|  | 16.8 The 16-lb bowling ball is cast horizontally onto a lane such that initially ω=0 and its mass center has a velocity v = 8 ft/s. If μk = 0.12, determine the distance the ball travels before it rolls without slipping. Neglect the finger holes and assume the ball has a uniform density. Ans: s = 4.06 ft |
|  | 16.9 A cord C is wrapped around each of the two 10-kg disks. If they are released from rest, determine the tension in the fixed cord D. Neglect the mass of the cord. Ans: Ay = 118 N |