EGR 280 – Mechanics Problem Set 6

Graphics and problem statements © 2004 R.C. Hibbeler.

Published by Pearson Education, Inc., Upper Saddle River, NJ.

|  | 6.1 The stress–strain diagram for a steel alloy having an original diameter of 0.5 in. and a gauge length of 2 in. is given in the figure. Determine approximately the modulus of elasticity for the material, the load on the specimen that causes yielding, and the ultimate load the specimen will support. Ans: E = 40x103 ksi Py = 7.85 kip Pu = 15.0 kip |
| --- | --- |
|  | 6.2 A 60-m long steel wire must not stretch more than 48 mm when the tension in the wire is 6 kN. Knowing that E = 200 GPa, determine (a) the smallest diameter which may be selected for the wire, (b) the corresponding value of the normal stress Ans: - 6.91 mm - 160.0 MPa |
|  | 6.3 A nylon thread is subjected to a 2-lb tension. Knowing that E = 0.5 Mpsi and that the maximum allowable normal stress is 6 ksi, determine (a) the required diameter of the thread, (b) the corresponding percent increase in length of the thread. Ans: - 0.0206 in - 1.20% |
|  | 6.4 A 4-mm diameter aluminum wire is observed to stretch 25 mm when the tension in the wire is 400 N. Knowing that E = 70 GPa, determine (a) the length of the wire, (b) the stress in the wire. Ans: - 55.0 m - 31.8 MPa |