Statics of particles and rigid bodies

∑F = R = 0	and	∑MO = 0	MO = rO × F

g = 9.81 m/s2 = 32.2 ft/s2

Normal stress, deformation

σ = P/A	ε = δ/L		σ = Eε		δ = PL / AE

Friction

static equilibrium: 		f ≤ μsN ; 	if moving, f = μkN

belt friction: 			T2 = T1 eμβ

Particle kinematics

v = dx/dt	a = dv/dt = d2x/dt2 = v(dv/dx)

uniform rectilinear motion: 	x = x0 + v0t

uniformly accelerated rectilinear motion:

v = v0 + a0t		x = x0 + v0t + ½ a0t2	v2 = v02 + 2a0(x-x0)

relative motion:  		xB = xA + xB/A	vB = vA + vB/A		aB = aA + aB/A

intrinsic coordinates:  		v = v et		a = dv/dt et + v2/ρ en

Particle kinetics

Newton’s Second Law:		∑F = ma

intrinsic coordinates:		∑Ft = m dv/dt		∑Fn = m v2/ρ

work and energy:		TA + UAB = TB

kinetic energy:			T = ½mv2

work done by spring:		U = ½k δ2  ;  by weight:  -Wy

conservation of energy:		VA + TA = VB + TB

if not conserved, then:		VA + TA = VB + TB + energy lost

impact and momentum:		mv1 + ∫Fdt = mv2

coefficient of restitution:	e = (v´B-v´A)/(vA-vB) = v´B/A/vA/B

Rigid body kinematics – plane motion

translation:		vA = vB	aA = aB

rotation:		v = ωk × r 	a = αk × r – ω2r

uniform rotation: 	θ = θ0 + ω0t

uniformly accelerated rotation:

ω = ω0 + α0t		θ = θ0 + ω0t + ½ α0t2	ω2 = ω02 + 2α0(θ-θ0)

general plane motion:		vB = vA + ωk × rB/A	aB= aA + αk × rB/A – ω2rB/A

rolling:				v = ωr			a = αr

pulleys:				ω1r1= ω2r2

gears:				ω1r1= -ω2r2

mass moments of inertia:

parallel axis theorem: 	IO = IG + md2

radius of gyration: 	k2 = I / m

rod: IG = mL2 / 12	disk: IG = mr2 / 2

ring: IG = mr2 		sphere: IG = 2mr2 / 5

Rigid body kinetics – plane motion

Newton’s Second Law: 			∑F = ma	∑MG = IGαk

non-centroidal rotation: 			∑Mo = Ioαk

kinetic energy:				T = ½mvG2 + ½ IG ω2 ; 	non-centroidal rotation: T = ½ IO ω2

momentum:				mvG + IGω ;  non-centroidal rotation: Ioω