SECS Senior Design Proposal

Project title

Air Cooled micro-CHP unit

Sponsor

OU Clean Energy Research Center

Sponsor Liaison

| Jim Leidel Director of Clean Energy Systems Oakland University Clean Energy Research Center / OU INC | 248-648-4805 office 248-765-2027 mobile www.oakland.edu/energy www.oakland.edu/cerc |
| --- | --- |

Project Summary

This project involves a micro-CHP, or combined heat and power unit for residential applications.  The unique nature is that this will be the first stage of developing a low cost air cooled unit suitable for the residential U.S. market.

A small displacement, single cylinder engine will be provided, converted to run on natural gas (or propane for portable use for this project).  A multi-pole, permanent magnet direct current generator will be attached.  A condensing, exhaust gas heat recovery unit will be fabricated and attached.  This will be an exhaust gas to hot water heat exchanger.

A DC or AC power take off system will be designed and connected to the generator output.  This will allow us to run and test the CHP unit under load for acoustic and thermal performance.

Jim Leidel will assist with the design and provide the electrical DC and AC power take off system as well as assist in its assembly and connection to the microCHP unit.

Design Choices

1.  AC or DC power take off system.  This will be an engineering & economic decision.

2.  Build a natural gas fuel conversion kit, or buy off the shelf.

3.  Size of the engine (based on desired electrical output and engine power/torque curve)

Potential DC System Schematic - Hybrid System

Small

Natural Gas Engine

Permanent Magnet Multi-pole Generator

3PH

AC to DC Rectifier

Charge Controller

Battery Bank

48VDC

Solar Charge Controller

OutBack
Power

Inverter/ Charger

AC Distribution Panel

AC Loads

A rough task list might be as follows:

- remove fuel tank, muffler, carburetor, and pull start

- couple the DC generator to the engine

- install gaseous fuel conversion kit

- install exhaust gas heat exchanger

- fabricate engine base and sound insulated enclosure

- connect to DC power take off system (including battery / inverter setup)

- test the sound and thermal performance under load

Potential AC System Schematic - Hybrid System

Small

Natural Gas Engine

1PH Induction

Generator

Battery Bank

48VDC

Solar Charge Controller

OutBack
Power

Inverter/ Charger

AC Distribution Panel

AC Loads

AC System Schematic:  Induction generator, direct to grid

Small

Natural Gas Engine

1PH Induction

Generator

AC Distribution Panel

AC Loads